Contents

Research questions	2

a) What is an autopilot?	2

b) What are the inputs & outputs of Autopilot system onboard an airplane?	2

Inputs:	2

Outputs:	2

c) What would be the role of the pilot in an airplane equipped with an autopilot?	2

d) What is the difference between Autopilot & SAS?	3

e) What is the role of the onboard sensors like (GPS, gyroscopes, ...etc.)? give example.	3

Flight Mechanics review:	4

a) State the general rigid body dynamics (RBD) equations in 3D space	4

Assume a mass element at a rigid body, the velocity of this mass w.r.t fixed axes:	4

By using the calculations of linear momentum:	4

By using the calculations of angular momentum:	5

we are dealing with an aircraft which means Ixy=Iyz=0   symmetry cond.	6

Linear Velocities transformation:	7

Let’s apply the above transformation on the gravitational force of the airplane as following:	7

Angular velocities transformation:	7

b) Classify the upper equations into (Kinetics & Kinematics) equations	8

c) What are the assumptions introduced while deriving those equations?	8

d) State the set of equations added to the (RBD) equations to form the Fixed wing Airplanes (EOM)	8

e) Classify the airplanes EOM equations mathematically.	9

f) What is the difference between the (Body axes) and the (earth or inertial axes)?	9

g) What is the difference between the pitch angle (θ) and the angle of attack (α), and between the sideslip angle (β) and the heading angle (ψ)?	9

Numerical solutions of ODEs	10

Some numerical solving algorithms for ODE	10

The chosen method for solving the aircraft EOM.	10

The general solution of 4th order Runge-Kutta method	11

Appendix: MATLAB code	13

# Autopilot literature review

# Research questions

## a) What is an autopilot?

An autopilot is a system that allows an airplane, marine vehicle, or spaceship to navigate without the need for constant manual control by a human operator. Human operators are not replaced by autopilots. Instead, the autopilot supports the driver in maintaining vehicle control.

## b) What are the inputs & outputs of Autopilot system onboard an airplane?

### Inputs:

#### Control Inputs:

- Radio Control Receivers

- MAVLink Data Streams, i.e., ground control stations or companion computers

#### Sensor inputs:

GPS, Compass, Airspeed, Rangefinders, IMU

#### Power Management Unit Inputs:

- Received Signal Strength Input (RSSI)

- Analog Airspeed Sensors

### Outputs:

- ESCs (electronic circuit that controls and regulates the speed of an electric motor) for motors

- Servos for control surfaces

- Telemetry data

- Actuators and General Purpose I/O like LEDs, buzzers etc

## c) What would be the role of the pilot in an airplane equipped with an autopilot?

The pilot sets the flight plan and turn on the autopilot sometimes the pilot reprograms the autopilot in case if worked incorrectly. autopilot is not smart enough to fly a plane by It self

## d) What is the difference between Autopilot & SAS?

SAS (Stability augmentation system) generally used during low and slow maneuvering where the pilot may be making constant attitude changes in preparation for landing.

Autopilot do same functions as SAS in addition it provides more functions, Autopilot Is more sophisticated than SAS

## e) What is the role of the onboard sensors like (GPS, gyroscopes, ...etc.)? give example.

Sensors provide the autopilot computers with data like speed, coordinates, position so the computers can estimate the states and give the correct control actions to the actuators for example

Gyroscope an IMU supplies the autopilot with position data so if the airplane in incorrect position it gives the actuator signal so that it can adjust position

# Flight Mechanics review:

## a) State the general rigid body dynamics (RBD) equations in 3D space

### Assume a mass element at a rigid body, the velocity of this mass w.r.t fixed axes:

V= Vc+ drdt         (1)

### By using the calculations of linear momentum:

The force at this point is given by:

δF= δmdVdt           (2)

Sub. By (1) in (2) and take summation for all point masses is in the rigid body:

F= δF= ddt (Vc+ drdt)δm

F=mdVcdt+ ddtdrdt δm= mdVcdt+d2dt2rδm

Since: rδm=0

∴F= mdVcdt

From general definition:

dAdtI= dAdtB + ω × A

So:

F= mdVcdt +m(ω × Vc)

Where:

F= Fxi+ Fyj+ Fzk,      ω=Pi+qj+rk,        Vc = ui+vj+wk

So:

Fx=m(u+qw-rv)

Fy=m(v+ru-pw)

Fz=mw+pv-qu

### By using the calculations of angular momentum:

δM= ddtδH= ddtr ×Vδm

0

By using the upper general definition:

drdtI= drdtB + ω × r

Then:            H= δH= r ×Vcδm+ r ×ω ×rδm

Where: r=xi+yj+zk

Then:

Hx=p(y2+ z2)δm-qxy δm-rxz δm

Hy=-pxyδm+q(x2+z)2 δm-ryz δm

Hz=-pxzδm+qyzδm-r(x2+y2)  δm

By expressing the above equations in terms of moment of inertia about body axes:

Hx=pIx-qIxy-rIxz

Hy=-pIxy+qIy-rIyz

Hz=-pIxz-qIyz+rIz

Since:

M= ddtH= dHdtB+ω ×H

L= Hx+qHz-rHy

M= Hy+rHx-pHz

N= Hz+pHy-qHx

### we are dealing with an aircraft which means Ixy=Iyz=0   symmetry cond.

Then: The airplane translational and rotational EOM are:

Fx=m(u+qw-rv)

Fy=m(v+ru-pw)

Fz=mw+pv-qu

L=Ixp-Ixzr+qrIz-Iy-Ixzpq

M=yq+rpIx-Iz+Ixzp2-r2

N=-Ixzp+Izr+pqIy-Ix+Ixzqr

By adding the calculations of earth axis to body axis transformation:

X1Y1Z1= cosψsinψ0-sinψcosψ0001 XEYEZE

F1=R3ψFE

X2Y2Z2=cosθ0-sinθ010sinθ0cosθ X1Y1Z1

F2=R2θF1= R2θR3ψFE

XBYBZB= 1000cosϕsinϕ0-sinϕcosϕ X2Y2Z2

FB=R1ϕF2= R1ϕR2θR3ψFE

### Linear Velocities transformation:

XYZE= CθCψSϕSθCψ-CϕSψCϕSθCψ+SϕSψ CθSψSϕSθSψ+CϕCψCϕSθSψ-SϕCψ-SθSθCθCϕCθuvwB

### Let’s apply the above transformation on the gravitational force of the airplane as following:

FGravityB= 1000cosϕsinϕ0-sinϕcosϕcosθ0-sinθ010sinθ0cosθcosψsinψ0-sinψcosψ000100mgE

FGravityB= -mgsinθmgsinϕcosθmgsinθcosϕB

Then:

-mgsinθ+FTX=m(u+qw-rv)

mgsinϕcosθ+FTY =m(v+ru-pw)

mgsinθcosϕ + FTZ=mw+pv-qu

### Angular velocities transformation:

| Ψ=ΨK1=ΨK2 | θ=θj2=θj3 | Φ=Φi3=Φi |
| --- | --- | --- |

ω=Ψ+θ+Φ=Pi+Qj+Rk=ΨK2+θj3+Φi
i3j3k3=1000cosΦ-sinΦ0sinΦcosΦijk

j3=cosΦj-sinΦk

k3=sinΦj+cosΦk
k2=-sinθi+cosθsinΦj+cosΦk=-sinθi+cosθsinΦj+cosθcosϕk
ω=Pi+Qj+Rk=Ψ+θ+Φ=Ψk2+θj3+Φi

=Ψ(-sinθi+cosθsinΦj+cosθcosϕk) +θcosΦj-sinΦk+Φi

∴PQR= 10-sinθ0cosϕcosθsinϕ0-sinϕcosϕcosθϕθψ

## b) Classify the upper equations into (Kinetics & Kinematics) equations

| Kinetics equations | Kinematics equations |
| --- | --- |
| -mgsinθ+FTX=m(u+qw-rv) mgsinϕcosθ+FTY =m(v+ru-pw) mgsinθcosϕ + FTZ=mw+pv-qu L=Ixp-Ixzr+qrIz-Iy-Ixzpq M=yq+rpIx-Iz+Ixzp2-r2 N=-Ixzp+Izr+pqIy-Ix+Ixzqr | PQR= 10-sinθ0cosϕcosθsinϕ0-sinϕcosϕcosθϕθψ XYZE= CθCψSϕSθCψ-CϕSψCϕSθCψ+SϕSψ CθSψSϕSθSψ+CϕCψCϕSθSψ-SϕCψ-SθSθCθCϕCθuvwB |

## c) What are the assumptions introduced while deriving those equations?

The assumptions are:

- The body is rigid which means the mass is constant and there is no change in geometry (Stress calculations are neglected).

- Fix a body axis at the center of mass of the rigid body.

## d) State the set of equations added to the (RBD) equations to form the Fixed wing Airplanes (EOM)

The additional EOM of airplanes:

→ The effect of the control surfaces: δe , δa , δr , δT

- The change in forces:

∆X=m ∂X∂u ∆u+∂X∂w ∆w+ ∂X∂δe ∆δe+ ∂X∂δT ∆δT

∆Y=m ∂Y∂v ∆v+∂Y∂β ∆β+ ∂Y∂δa ∆δa+ ∂Y∂δr ∆δr

∆Z=m ∂Z∂u ∆u+∂Z∂w ∆w+ ∂Z∂w ∆w+∂Z∂q ∆q+ ∂Z∂δe ∆δe+ ∂Z∂δT ∆δT

- The change in moments:

∆L=Ixx ∂L∂β ∆β+∂L∂p ∆p+∂L∂r ∆r+ ∂L∂δr ∆δr+ ∂L∂δa ∆δa

∆M=Iyy ∂M∂u ∆u+∂M∂w ∆w+ ∂M∂w ∆w+∂M∂q ∆q+ ∂M∂δe ∆δe+ ∂M∂δT ∆δT

∆N=Izz ∂N∂β ∆β+∂N∂p ∆p+∂N∂r ∆r+ ∂N∂δr ∆δr+ ∂N∂δa ∆δa

## e) Classify the airplanes EOM equations mathematically.

The EOMs are coupled, nonlinear and first order DE.

## f) What is the difference between the (Body axes) and the (earth or inertial axes)?

- Body axes: they are a set of axes which are fixed at the body in its translational and rotations.

- Inertial axes: they are a set of axes which are fixed at a specified position on the ground.

## g) What is the difference between the pitch angle (θ) and the angle of attack (α), and between the sideslip angle (β) and the heading angle (ψ)?

- Pitch angle: it’s formed due to rotation about y-axis.

- AOA: it’s formed due to a difference between the direction of flow and the plane wing.

- Sideslip angle: it’s formed due to due to lateral deviation between the plane and the direction of the flow.

- Yaw angle (heading): it’s formed due to due to the rotation about z-axis.

# Numerical solutions of ODEs

## Some numerical solving algorithms for ODE

one step methods, are methods that involve only yi and intermediate quantities to compute the next value yi+1. In contrast, multi-step methods use more than the previous point

One-step methods, like:

Euler-Cauchy method

Improved Euler method

Raunge-Kutta method

Backward Euler method

Multistep methods, like:

Adams-Bash forth method

Adams-Moulton method

Other methods

Predictor-Corrector Methods

Exponential integrator methods

## The chosen method for solving the aircraft EOM.

We choose to use Raunge-Kutta method since its one of the most accurate one step methods used in the numerical solutions, in addition it’s the method used in the MATLAB ODE45 algorithm.

The order of the Raunge-Kutta indicates the number of slopes used in the approximation; so higher orders mean higher accuracy of the solution; the wights of the slopes is determined comparing the coefficients with Taylor expansion of the function.[1] anyway, the fourth order Raunge-Kutta is commonly as it gives good accuracy and acceptable computational power.

Initial conditions:

u0, v0, w0,x0, y0, z0, θ0, φ0, ψ0, p0, q0, r0

Inputs:

δa, δe, δr, δT

Outputs:

Components of air craft velocity: u , v , w

Position: x , y , z

Euler angles: θ , φ , ψ

Angular velocity components: p , q , r

[1] for more details regarding the derivation of Raunge-Kutta method use the following link

https://www.math.hkust.edu.hk/~machas/numerical-methods-for-engineers.pdf?fbclid=IwAR1mOjc35AS3QV_ZuL0Z_nV8Ph3nCop4FlsaS_ueBMz31whmOBPh9LJ2GlM

## The general solution of 4th order Runge-Kutta method

k1=Δtftn,xn

k2=Δtftn+12Δt,xn+12k1

k3=Δtftn+12Δt,xn+12k2

k4=Δtftn+Δt,xn+k3

xn+1=xn+16k1+2k2+2k3+k4

For the following system of ODEs

f=dy1dt=sin⁡(t)+cos⁡y1+sin⁡y2g=dy2dt=cos⁡(t)+sin⁡y2

IC @t=0,      y1=0, y2=0            tf=20 sec            n=100               h=Δtn

| K1=hftn,yn1,yn2 | q1=hgtn,yn1,yn2 |
| --- | --- |
| K2=hftn+12h,yn1+12K1,yn2+12q1 | q2=hgtn+12h,yn1+12K1,yn2+12q1 |
| K3=hftn+12h,yn1+12K2,yn2+12q2 | q3=hgtn+12h,yn1+12K2,yn2+12q2 |
| K4=hftn+h,yn1+K3,yn2+q3 | q4=hgtn+h,yn1+K3,yn2+q3 |

yn+11=yn1+16q1+2q2+2q3+q4

yn+12=yn2+16k1+2k2+2k3+k4

# Appendix: MATLAB code

clc clear All close All t1=0; t2=20; n=100; h=(t2-t1)/n; t=(linspace(t1,t2,n+1))'; y1=zeros(length(t),1); y2=zeros(length(t),1); y2(1)=1; y1(1)=-1; for n=1:length(t)-1 k1=h*(cos(t(n))+sin(y2(n))); q1=h*(sin(t(n))+cos(y1(n))+sin(y2(n))); k2=h*(cos(t(n)+0.5*h)+sin(y2(n)+0.5*k1)); q2=h*(sin(t(n)+0.5*h)+cos(y1(n)+0.5*q1)+sin(y2(n)+0.5*k1)); k3=h*(cos(t(n)+0.5*h)+sin(y2(n)+0.5*k2)); q3=h*(sin(t(n)+0.5*h)+cos(y1(n)+0.5*q2)+sin(y2(n)+0.5*k2)); k4=h*(cos(t(n)+h)+sin(y2(n)+k3)); q4=h*(sin(t(n)+h)+cos(y1(n)+q3)+sin(y2(n)+k3)); y2(n+1)=y2(n)+(1/6)*(k1+2*k2+2*k3+k4); y1(n+1)=y1(n)+(1/6)*(q1+2*q2+2*q3+q4); end figure hold on plot(t,y1,'r') plot(t,y2,'b') legend({'y(1)','y(2)'},... 'Location','southeast','FontSize',8,'Interpreter','latex') xlabel('$t$','Interpreter','latex','FontSize',13) %% Check using ode45 [tv,Yv]=ode45(@sys_fun,[0 20],[-1 1]); plot(tv,Yv(:,1),'r--','LineWidth',1.5) plot(tv,Yv(:,2),'k--','LineWidth',1.5) legend({'y(1)','y(2)','y(1)ode45','y(2)ode45'},... 'Location','southeast','FontSize',8,'Interpreter','latex') xlabel('$t$','Interpreter','latex','FontSize',13) function f=sys_fun(t,Y) f(1,1)=sin(t)+cos(Y(1))+sin(Y(2)); f(2,1)=cos(t)+sin(Y(2)); end